Extending the Tutorial: Integrating Python Coding and Machine Learning Into Your Electronics Cooling CFD Workshop

This guide outlines how to seamlessly integrate Python programming and introductory machine learning concepts into your current electronics cooling workshop—without introducing an entirely separate module. The strategy is to embed coding naturally within the CFD workflow, showing how Python can enhance data analysis, automate tasks, and enable design innovations with surrogate modeling.

1. Python for Post-Processing and Visualization (During or After CFD)

Objective: Teach students to extract, process, and plot simulation results using Python rather than only relying on Fluent’s GUI.

How to Integrate:

- Show how to export simulation outputs (e.g., temperature, velocity, heat flux data) from ANSYS Fluent (as CSV or via Python APIs such as PyFluent).

- Use Jupyter notebooks so students can analyze the exported results with Python code.

Content Example:

- Guide students through a simple script to read simulation data and plot:

  - Average and maximum base plate temperatures versus fin design

  - Temperature distribution across the base plate

  - Comparative bar charts for baseline, rectangular, and triangular fins

import pandas as pd
import matplotlib.pyplot as plt

# Example: Load exported Fluent CSV results
df = pd.read_csv('fluent_results.csv')

# Plot temperature rise for different geometries
plt.bar(df['design'], df['avg_temp'])
plt.xlabel('Heat Sink Design')
plt.ylabel('Average Base Plate Temperature (°C)')
plt.title('Comparison of Cooling Performance')
plt.show()

Key Learning:

- Students see how coding can rapidly process multiple results for deeper design insights, reinforcing both simulation and data literacy.

2. Automating Parametric Studies With Python (Within Your Workshop Tasks)

Objective: Demonstrate how Python can automate time-consuming, repetitive CFD tasks (e.g., batch runs of different fin heights or spacings).

How to Integrate:

- Show that instead of manually changing parameters in the GUI, you can leverage Python (PyFluent or Ansys Workbench scripting) to automate geometry generation, simulation runs, and data collection.

- For education, focus on scripting post-processing or automatically renaming/saving runs for later analysis.

Content Example:

- Provide starter Python scripts (or pseudo-code) that demonstrate:

  - Looping through varying fin geometries

  - Saving each simulation’s outcome to a results file

fin_heights = [10, 20, 30]  # mm
avg_temps = []

for height in fin_heights:
    # (Pseudocode) Update geometry with new height, run simulation in Ansys Fluent, export temperature
    result = run_simulation_with_fin_height(height)
    avg_temps.append(result['avg_temp'])

# Plot results
plt.plot(fin_heights, avg_temps, marker='o')
plt.xlabel('Fin Height (mm)')
plt.ylabel('Average Plate Temperature (°C)')
plt.title('Effect of Fin Height on Cooling Performance')
plt.show()

3. Design of Experiments (DOE) & Surrogate Modeling With Python/Machine Learning

Objective: Introduce the basic concepts of surrogate modeling (e.g., using regression or neural networks to predict CFD outputs) within the context of engineering design.

How to Integrate:

- During your free exploration/design optimization phase, ask students to perform a small parametric sweep (e.g., try three fin heights and three fin spacings—nine experiments total).

- After running different CFD cases, aggregate the results into a CSV suitable for Python analysis.

- In Python, teach students to:

  - Fit a simple regression model (start with linear regression; advanced students might try scikit-learn’s decision tree or neural network)

  - Make rapid predictions for new designs (interpolative use, not high-dimensional black-box)

Content Example:

- Example in a Jupyter notebook:

import pandas as pd
from sklearn.linear_model import LinearRegression
import numpy as np

# Load the design and result data
data = pd.read_csv('doe_results.csv')  # Columns: fin_height, fin_spacing, avg_temp

X = data[['fin_height', 'fin_spacing']]
y = data['avg_temp']

# Train a regression model
model = LinearRegression()
model.fit(X, y)

# Predict for new design (e.g., height=25mm, spacing=15mm)
new_design = np.array([[25, 15]])
predicted_temp = model.predict(new_design)
print(f"Predicted avg temp for new design: {predicted_temp[^0]:.2f} °C")

- Briefly mention more advanced surrogate models (random forests, neural nets) possible for larger datasets or as next-step projects.

Learning Outcome:

- Students appreciate that once a surrogate model is trained, they can predict cooling performance for many design scenarios in seconds, not hours—showing the value of data-driven engineering and opening the door to machine learning.

4. Introducing Machine Learning Concepts in Context

Keep it practical and tightly linked to the CFD workflow:

- Explain the purpose:

  - Machine learning surrogates can approximate expensive CFD runs

  - Useful for rapid trade-off analysis and optimization

- Choose accessible examples:

  - Regression for predicting temperature as a function of fin geometry

  - Classification for “adequate” vs “inadequate” cooling (if you wish)

- Use small datasets students generate themselves

Advanced Note: For motivated groups, show how neural networks (e.g., using scikit-learn’s MLPRegressor) or graph neural networks (for field or spatial prediction, e.g., mesh-based output) can accelerate optimization.

5. Optional: Visualization and Reporting With Python

- Automate the generation of result tables, plots, and even slide decks directly from simulation data using matplotlib, seaborn, or simple report generation tools.

- For open-source demonstration, use Jupyter notebook as the central platform to run, visualize, and comment on the entire workflow.

Summary Table: Seamless Integration Ideas

| Workshop Phase | Extension via Python/Machine Learning | Example Content/Task |
| --- | --- | --- |
| Post-processing | Python plotting of results | Bar plots of temperatures per design |
| Parametric exploration | Scripting automated runs or batch analysis | Batch run results plotted as response surfaces |
| Design optimization | Regression/surrogate modeling | Fit regression model: predict best fin height/spacing |
| Reporting/presentation | Automated report or figure generation | Create summary slides/figures from all batch simulation data |

Implementation Tips

- Provide students with starter notebooks/scripts and encourage them to modify and explore.

- Keep datasets small—focus on pedagogy, not model depth.

- For non-CS students, focus more on using and modifying code rather than developing from scratch.

- Highlight connections to real engineering workflow (industry digitalization, data-driven design).

By blending Python and machine learning directly into the existing CFD workflow, your students will gain practical coding, data analysis, and emerging AI skills as natural extensions of their simulation-based learning—without fragmenting the pedagogical flow of your workshop.

⁂

- https://www.ansys.com/blog/open-source-access-to-fluent-with-pyfluent

- https://fluent.docs.pyansys.com/version/stable/getting_started/faqs.html

- https://hammer.purdue.edu/articles/thesis/b_Machine-Learning-Aided_Development_of_Surrogate_Models_for_Flexible_Design_Optimization_of_Enhanced_Heat_Transfer_Surfaces_b/28373483

- https://arxiv.org/abs/2509.06041

- https://www.cfd-online.com/Forums/fluent/259087-exporting-solution-data-fluent-using-ansys-fluent-core-python-script.html

- https://asmedigitalcollection.asme.org/InterPACK/proceedings/InterPACK2022/86557/V001T01A016/1153366

- https://pmc.ncbi.nlm.nih.gov/articles/PMC11368675/

- https://fluent.docs.pyansys.com/version/stable/examples/index.html

- https://asmedigitalcollection.asme.org/IMECE/proceedings/IMECE2024/88599/V001T02A025/1211537

- https://www.nature.com/articles/s41598-024-73380-1

- https://blog.ozeninc.com/resources/success-stories-pyfluent-case-studies-you-should-know

- https://www.sciencedirect.com/science/article/pii/S1359431122018476

- https://pubs.aip.org/aip/jap/article/138/6/064503/3358330/Multi-objective-optimization-of-porous

- https://www.sciencedirect.com/topics/engineering/latin-hypercube-sampling

- https://www.sciencedirect.com/science/article/abs/pii/S0360132325011382

- https://engys.com/blog/automating-cfd-analysis-tasks-with-python-and-helyx/

- https://www.sciencedirect.com/science/article/pii/S1290072925002194

- https://www.sciencedirect.com/science/article/pii/S0045782525001938

- https://www.symscape.com/news/automate-cfd-simulations-with-python-scripting.html

- https://onlinelibrary.wiley.com/doi/abs/10.1002/qre.1924

- https://arxiv.org/html/2509.01963v1

- https://www.cfdsource.com/ShowBlog/automating-cfd-python/

- https://www.sciencedirect.com/science/article/abs/pii/S2214860421006011

- https://ieeexplore.ieee.org/document/10700243/

- https://pubs.aip.org/aip/pof/article/34/8/081906/2846694/Deep-learning-based-surrogate-model-for-three

- https://www.reddit.com/r/CFD/comments/f5rbfh/best_software_for_automated_model_generation_with/

- https://digitalcommons.usf.edu/cgi/viewcontent.cgi?article=8366&context=etd

- https://www.linkedin.com/pulse/neural-network-based-surrogates-computational-fluid-adarsh-yiepc

- https://docs.flexcompute.com/projects/flow360/en/release-23.1.1.0/quickStart/PythonAPI_AutomatedMeshing/PythonAPI_AutomatedMeshing.html

- https://arc.aiaa.org/doi/10.2514/6.2004-2011

- https://caeassistant.com/blog/python-cae-automation/

- https://semiengineering.com/boost-your-cfd-workflow-productivity-with-fidelity-python-api-part-1/

- https://www.hellotriangle.io/articles/parametric-mesh-modeling

- https://fluent.docs.pyansys.com

- https://arxiv.org/abs/2208.09683

- https://www.sciencedirect.com/science/article/pii/S030626192300435X

- https://innovationspace.ansys.com/courses/courses/getting-started-with-pyfluent/lessons/lesson-6-using-pyfluent-visualization/

- https://www.sciencedirect.com/science/article/abs/pii/S1359431122018476

- https://arxiv.org/html/2510.03261v1