Teaching Neural Network–Based Surrogates in CFD: A Class Project Using Heat Sink Design in ANSYS Fluent

This topic blends computational fluid dynamics (CFD) with machine learning (ML) to create surrogate models that can accelerate engineering design. A class project on heat sink optimization provides an excellent, hands-on way to teach and apply these ideas.

1. Learning Objectives

By the end of this project, students should be able to:

- Explain what a surrogate model is and why it is used in CFD.

- Run a baseline CFD simulation of a heat sink in ANSYS Fluent.

- Extract CFD data (e.g., temperature, pressure drop, or thermal resistance) for varying design parameters.

- Train a neural network regression model to predict key CFD outputs.

- Validate the surrogate model’s accuracy and use it for design optimization.

2. Background Concepts to Cover in Lecture

- Heat Transfer Basics: Conduction, convection, and thermal resistance in heat sinks.

- Computational Fluid Dynamics: Governing equations for fluid flow and heat transfer (Navier–Stokes and energy equations).

- Design Parameters: Fin height, fin spacing, number of fins, base thickness, inlet velocity, and heat source power.

- Surrogate Modeling: Definition, advantages (speed), and limitations (accuracy).

- Neural Networks: Input features, hidden layers, nonlinearity, training, overfitting, validation.

- Performance Metrics: Mean squared error (MSE), coefficient of determination (R2), and prediction vs. simulation plots.

3. Project Workflow

Step 1 – Setup CFD Simulations in ANSYS Fluent

- Base geometry: a simple plate-fin heat sink with a heat source on the base and forced-air convection.

- CFD tasks:

  - Import or build geometry

  - Generate mesh (check skewness < 0.8)

  - Define materials (Aluminum for sink, Air for fluid)

  - Set energy and turbulence models (e.g., k–ε)

  - Apply boundary conditions (inlet velocity and temperature, outlet pressure, wall heat flux)

  - Solve until convergence (residuals < 10⁻⁴)

  - Post-process results (average surface temperature, total heat transfer rate)

Step 2 – Design of Experiments (DOE)

Create a parametric study by varying 3–4 input variables (e.g., fin height, number of fins, base thickness, and fan velocity). Generate a dataset of 20–40 CFD runs.

Step 3 – Build the Surrogate Model

- Use Python with TensorFlow or PyTorch.

- Inputs: design parameters.

- Outputs: temperature or thermal resistance.

- Split dataset: 80% training, 20% testing.

- Train using mean squared error loss.

- Evaluate performance and visualize predicted vs. CFD results.

Example Python snippet:

import tensorflow as tf
from sklearn.model_selection import train_test_split
from sklearn.preprocessing import StandardScaler

# X: design parameters, y: CFD results
X_train, X_test, y_train, y_test = train_test_split(X, y, test_size=0.2)
scaler = StandardScaler().fit(X_train)
X_train = scaler.transform(X_train)
X_test = scaler.transform(X_test)

model = tf.keras.Sequential([
    tf.keras.layers.Dense(64, activation='relu', input_shape=(X_train.shape[^1],)),
    tf.keras.layers.Dense(64, activation='relu'),
    tf.keras.layers.Dense(1)
])
model.compile(optimizer='adam', loss='mse', metrics=['mae'])
model.fit(X_train, y_train, epochs=200, validation_split=0.2)

Step 4 – Optimization with the Surrogate

Once the neural network is trained, it can rapidly evaluate thousands of geometries to find an optimal design that minimizes temperature or thermal resistance under given constraints.

4. Example Assessment or Deliverables

- Part A: CFD Simulation Report (geometry, mesh, setup, results)

- Part B: Dataset Description (inputs/outputs, ranges)

- Part C: Neural Network Training and Validation Report

- Part D: Optimization Results and Discussion

- Part E: Reflection on speed-up vs. fidelity trade-offs

5. Suggested References

- Machine Learning Based Surrogate Models for Microchannel Heat Sink Optimization (arXiv:2208.09683)

- Song et al., Surrogate-Based Analysis and Optimization for the Design of Heat Sinks with Jet Impingement (IEEE TCPMT, 2014)

- Karam Almasri, Electronics Cooling Using a Heat Sink – ANSYS Fluent Tutorial (YouTube, 2023)

Optional Extension

Have students compare different ML models:

- Random Forests, Gradient Boosting, Neural Networks.

- Use SHAP values to explain feature importance.

- Integrate the surrogate in a multi-objective optimization (e.g., minimize thermal resistance and pressure drop).

If you share your class level (undergraduate/graduate) or course type (CFD, ML, or thermal design), I can simplify or expand the exercises and rubrics accordingly.

⁂

- https://innovationspace.ansys.com/courses/courses/intro-to-heat-transfer-in-fluids/lessons/simulation-examples-homeworks-and-quiz/

- https://www.sciencedirect.com/science/article/abs/pii/S0378778824011368

- https://arxiv.org/pdf/2208.09683.pdf

- https://elib.dlr.de/198703/1/bokil2023DLRK_published.pdf

- https://innovationspace.ansys.com/knowledge/forums/topic/ansys-fluent-student-conjugate-heat-transfer-in-a-heat-sink/

- https://pubs.aip.org/aip/pof/article/34/8/081906/2846694/Deep-learning-based-surrogate-model-for-three

- https://www.sciencedirect.com/science/article/pii/S259012302400255X

- https://laccei.org/LACCEI2024-CostaRica/papers/Contribution_1795_final_a.pdf

- https://arxiv.org/html/2411.03598v1

- https://www.youtube.com/watch?v=Nii9WbjPD44

- https://cfd.ninja/ansys-fluent/ansys-fluent-tutorial-heatsink/

- https://arxiv.org/abs/2208.09683

- https://www.utdallas.edu/~jiezhang/Journals/JIE_2014_IEEE_TCPMT_SBO.pdf

- https://docs.nrel.gov/docs/fy24osti/84235.pdf

- https://mavmatrix.uta.edu/cgi/viewcontent.cgi?article=2029&context=mechaerospace_theses

- https://www.youtube.com/watch?v=yPlSPHtOF1U

- https://www.neuralconcept.com/post/applying-machine-learning-in-cfd-to-accelerate-simulation